# RESUMO (14, N)

FILTRAÇÃO EM MÚLTIPLAS ETAPAS APLICADA AO TRATAMENTO DE ÁGUA COM PRESENÇA DE ALGAS: AVALIAÇÃO DE VARIÁVEIS OPERACIONAIS (12, N, Espaço 1)

A Filtração em Múltiplas Etapas (FiME) é uma seqüência de tratamento que é formada por sistemas de pré-tratamento em pedregulho e filtração lenta. Existem vários estudos que comprovam a eficiência dessa tecnologia no tratamento de água com alta turbidez, o que proporciona uma considerável ampliação nos limites de aplicabilidade da filtração lenta. Porém, poucos são os estudos realizados com a utilização da FiME no tratamento de água com elevados teores de algas.

O presente trabalho foi desenvolvido, em escala piloto, com a utilização da filtração em múltiplas etapas no tratamento de água do Lago Paranoá (Brasília-DF), tendo como objetivo geral .................... Como objetivos específicos, o trabalho procurou verificar .........

A instalação piloto utilizada no experimento é formada por um pré-filtro dinâmico de pedregulho, que foi operado com taxa de 36 m3/m2.dia, dois pré-filtros de pedregulho com escoamento ascendentes, de mesma granulometria, nos quais foram testadas a adoção de taxas de filtração de 12 e 18 m3/m2.dia, e dois filtros lentos de areia, os quais foram operados com taxas de 3 e 6 m3/m2.dia.

Durante a realização do estudo a água bruta apresentou  valores de turbidez variando de 2,5 a 11 uT, e concentrações de clorofila-a na faixa de 1 a 49 g/L, e valor médio de 25,8 g/L. O desempenho de cada unidade, e do sistema de tratamento como um todo, foi avaliado a partir de dados de....................

Os resultados sugerem que a FiME possui um grande potencial no tratamento de águas com elevadas concentrações de algas, produzindo efluentes com teores de clorofila-a abaixo de 0,5 g/L e turbidez sempre inferior a 1 uT.

O aumento da taxa de filtração nos pré-filtros ascendentes em camadas influenciou significativamente ..............

Os dois filtros lentos produziram água com qualidade similar, independentemente da condição operacional ou do arranjo físico do sistema de pré-tratamento. Entretanto, a perda de carga foi maior quando o filtro lento recebia água com qualidade inferior.

# ABSTRACT

MULTISTAGE FILTRATION APPLIED TO THE TREATMENT OF ALGAL LADEN WATERS: EVALUATION OF OPERATIONAL CONDITIONS

Multistage Filtration (MSF) is a treatment sequence based on the combination of gravel roughing filtration and slow sand filtration. There are many studies that show that this technology is very efficient in treating water with high values of turbidity, extending the application of the slow sand filtration to waters of poorer quality.  However, very few studies have investigated the use of MSF in the treatment of waters with high concentrations of algae.

The present work was developed in a multistage filtration pilot plant treating water from Paranoá Lake (Brasília - DF - Brazil). Considering the particularities of the algal laden waters, the general objective of the investigation was……. More specifically, the work evaluated the……….

The pilot plant comprises a dynamic roughing filter, operated at a constant filtration rate of 36 m3/m2.day, two similar five layers upflow rough filters and two slow sand filters. Two filtration rates were applied to the upflow filters, 12 e 18 m3/m2.day, as well as to the slow sand filters, 3 e 6 m3/m2.day.

During the experiments the raw water presented turbidity varying from 2.5 to 11 NTU and chlorophyll-a concentration in the range of 1 to 49 g/L, with an average value of 25.8 g/L. The performance of………….

The results show that the MSF system tested is able to treat waters with high algae concentrations and low turbidity, producing water with less than 0.5 g/L chlorophyll-a and turbidity always inferior to 1 NTU.

The increase of the filtration rate applied to the upflow rough filter from 12 to 18 m3/m2.day  influenced negatively the performance of this unit. On the other side, the use of a weekly hydraulic cleaning instead of a sole cleaning procedure at the end of the filtration run, does not seem to influence the chlorophyll-a and turbidity removal efficiencies.

Regardless the operational condition adopted and the pre-treatment arrangement, the two slow sand filter units always produced water with very similar quality. However, the head loss development was higher in the unit which was fed with poorer water quality.